# IoT Midterm Presentation Script

Motion-Adaptive AI Companion Robot
LLM-based Emotional Interactive AIoT System

Team: Park Hyodam (202304709, Captain) · Jo Hyeonjin (202304438)

Slides: 아이오티 중간발표 체크.pptx (10 slides)

Target time: ~3 minutes (+ ~2 min Q&A)

## Part 1 — English Script (3 minutes)

[Slide 1 — Title · 0:00–0:15]

Good morning/afternoon, Professor.

We are Team Park Hyodam and Jo Hyeonjin from IoT Systems.

Our project is Motion-Adaptive AI Companion Robot — an LLM-based Emotional Interactive AIoT System on Raspberry Pi.

[Slide 2 — A. Main Idea & Objective · 0:15–0:40]

Our main idea is a desktop companion robot that senses the user and responds emotionally.

We build one AIoT pipeline on a single device: Sensor → Decision → Actuator.

For sensing, we use a camera and an ultrasonic sensor. For output, we use LCD, LED, and buzzer.

For interaction, we show short English replies on a 2×16 LCD using keywords or LLM.

Our four objectives are: build the pipeline, detect motion and proximity, show state on LCD and LED, and support short English chat.

[Slide 3 — B. System Design & Methodology · 0:40–1:05]

Our system has three modules.

Input: camera, ultrasonic HC-SR04, and terminal keyboard.

Processing: companion.py as the main loop, plus motion.py and reply.py.

Output: I2C LCD at address 0x27, LED on GPIO 17, and buzzer on GPIO 27.

Every two seconds, we capture a frame with rpicam-still.

detect_motion() checks the center ROI frame difference.

The ultrasonic sensor checks if distance is 20 cm or less.

If motion OR proximity is detected, we call show_motion().

[Slide 4 — C. Key Code · 1:05–1:30]

The core class is Actuator in lcd_actuator.py.

show_idle() means waiting — LED off, LCD shows "Companion Bot" and "Zzz...".

show_motion() means user detected — LED on, greeting on LCD, and a 0.4-second buzzer beep.

show_reply() prints a two-line chat response on the LCD.

We started from a simpler draft in led_lcd.txt, and we have now integrated the buzzer into show_motion().

[Slide 5 — D. Hardware · 1:30–1:50]

Our hardware is Raspberry Pi 4 with five peripherals.

The camera uses CSI and rpicam-still. The LCD uses I2C.

The ultrasonic uses TRIG on GPIO 16 and ECHO on GPIO 18.

The LED is on GPIO 17. The buzzer is on GPIO 27.

We had a pin conflict because ECHO uses pin 18, so we moved the buzzer from 18 to 27.

In Lab 9, we also practiced GPIO output with a touch sensor and dual LEDs.

[Slide 6 — E. Current Progress · 1:50–2:10]

Here is our current progress.

Completed: LCD, LED, and buzzer control; ultrasonic detection; OpenCV motion detection; and basic states — idle, motion, and reply.

In progress: companion.py integration testing and motion threshold tuning.

To do: emotional reply with LLM or keywords, state-based reactions for approach, stay, and alert, plus error handling and backup mode.

[Slide 7 — F. Project Understanding · 2:10–2:30]

Our final system is not just motion detection.

We plan a state flow: Idle when no user is detected, Approach when distance is within 20 cm,

Stay when the user remains nearby and we ask "Want to talk?", Alert for sudden large motion, and Return to idle after timeout.

The key point is: we classify the user situation and choose an emotional response.

[Slide 8 — Challenges & Solutions · 2:30–2:45]

We faced five challenges.

GPIO pin conflict — solved by moving buzzer to GPIO 27.

No speaker — we use LCD text only.

Local LLM too slow on Pi — we use keyword replies plus optional Gemini API.

Motion false triggers — we use ROI, threshold tuning, and a 3-second cooldown.

[Slide 9 — Future Plan · 2:45–2:55]

Our roadmap has three steps.

Step 1: stabilize sensors and actuators.

Step 2: refine motion states — idle, approach, stay, and alert.

Step 3: add emotional reply logic on the LCD.

Our final target is: motion and distance sensing → emotional state → LCD, LED, buzzer, and short LLM response.

[Slide 10 — Thank You · 2:55–3:00]

That is our midterm draft presentation.

Thank you. We are happy to take your questions.

### English — Timing Guide

| Slide | Topic | Time |
| --- | --- | --- |
| 1 | Title | 0:15 |
| 2 | Main Idea | 0:25 |
| 3 | System Design | 0:25 |
| 4 | Key Code | 0:25 |
| 5 | Hardware | 0:20 |
| 6 | Progress | 0:20 |
| 7 | State Flow | 0:20 |
| 8 | Challenges | 0:15 |
| 9 | Future Plan | 0:10 |
| 10 | Thank You | 0:05 |
| Total |  | ~3:00 |

## Part 2 — 한국어 대본 (3분)

[슬라이드 1 — 제목 · 0:00–0:15]

안녕하세요, 교수님. IoT Systems 수업 중간 발표입니다.

팀장 박효담, 팀원 조현진입니다.

프로젝트 제목은 Motion-Adaptive AI Companion Robot, LLM 기반 감정형 AIoT 동반 로봇입니다.

[슬라이드 2 — A. 주제와 목표 · 0:15–0:40]

핵심 아이디어는 사용자를 감지하고 감정적으로 반응하는 데스크톱 동반 로봇입니다.

라즈베리파이 한 대에서 센서 → 판단 → 출력 AIoT 파이프라인을 구현합니다.

센서는 카메라와 초음파 센서, 출력은 LCD, LED, 버저, 상호작용은 2×16 LCD에 짧은 영어 응답을 표시합니다.

목표는 네 가지입니다. 파이프라인 구축, 움직임·근접 감지, LCD·LED 상태 표현, 짧은 영어 대화 지원입니다.

[슬라이드 3 — B. 시스템 설계 · 0:40–1:05]

시스템은 세 모듈로 나눴습니다.

입력: 카메라, 초음파 HC-SR04, 터미널 키보드.

처리: 메인 루프 companion.py, motion.py, reply.py.

출력: I2C LCD 0x27, LED GPIO 17, 버저 GPIO 27.

2초마다 rpicam-still로 프레임을 촬영하고, detect_motion()으로 중앙 ROI 프레임 차이를 확인합니다.

초음파로 20cm 이하 거리도 함께 검사합니다. 움직임 또는 근접이 감지되면 show_motion()을 호출합니다.

[슬라이드 4 — C. 핵심 코드 · 1:05–1:30]

핵심은 lcd_actuator.py의 Actuator 클래스입니다.

show_idle()은 대기 상태입니다. LED를 끄고 LCD에 "Companion Bot", "Zzz..."를 표시합니다.

show_motion()은 사용자 감지 상태입니다. LED를 켜고 인사말을 띄우며, 버저를 0.4초 울립니다.

show_reply()는 LCD 두 줄에 대화 응답을 출력합니다.

처음에는 led_lcd.txt 초안으로 LCD와 LED만 구현했고, 지금은 show_motion()에 버저까지 통합했습니다.

[슬라이드 5 — D. 하드웨어 · 1:30–1:50]

하드웨어는 라즈베리파이 4와 다섯 가지 주변장치입니다.

카메라는 CSI, LCD는 I2C, 초음파는 TRIG 16번·ECHO 18번, LED는 GPIO 17번, 버저는 GPIO 27번입니다.

ECHO가 18번이라 버저와 핀이 겹쳐서, 버저를 27번으로 분리했습니다.

Lab 9에서는 터치 센서와 LED 2개 제어로 GPIO 출력 경험도 확보했습니다.

[슬라이드 6 — E. 현재 진행 상황 · 1:50–2:10]

현재 진행 상황입니다.

완료: LCD·LED·버저 제어, 초음파 거리 감지, OpenCV 움직임 감지, idle·motion·reply 기본 상태.

진행 중: companion.py 통합 테스트, motion 임계값 튜닝.

예정: LLM·키워드 감정 응답, approach·stay·alert 상태 반응, 오류 처리·백업 모드.

[슬라이드 7 — F. 프로젝트 이해 · 2:10–2:30]

최종 시스템은 단순 움직임 감지가 아닙니다.

상태 흐름을 설계했습니다. Idle — 사용자 없음, Approach — 20cm 이하 접근,

Stay — 근처에 머무름, "Want to talk?" 유도, Alert — 갑작스러운 큰 움직임, Return — 입력 없으면 idle 복귀.

핵심은 사용자 상황을 상태로 분류하고 감정형 반응을 선택하는 것입니다.

[슬라이드 8 — 문제와 해결 · 2:30–2:45]

다섯 가지 문제와 해결입니다.

GPIO 핀 충돌 → 버저 27번으로 분리.

스피커 없음 → LCD 텍스트 응답.

Pi 로컬 LLM 과부하 → 키워드 + Gemini API 병행.

움직임 오탐 → ROI, 임계값, 3초 쿨다운.

[슬라이드 9 — 향후 계획 · 2:45–2:55]

로드맵은 세 단계입니다.

1단계: 센서·액추에이터 안정화.

2단계: idle·approach·stay·alert 상태 정교화.

3단계: LCD 감정형 응답 로직.

최종 목표는 움직임·거리 감지 → 감정 상태 → LCD·LED·버저·짧은 LLM 응답입니다.

[슬라이드 10 — 감사 · 2:55–3:00]

중간 발표는 여기까지입니다.

감사합니다. 질문 있으시면 답변드리겠습니다.

### 한국어 — 슬라이드별 시간표

| 슬라이드 | 내용 | 시간 |
| --- | --- | --- |
| 1 | 제목 | 0:15 |
| 2 | 주제·목표 | 0:25 |
| 3 | 시스템 설계 | 0:25 |
| 4 | 핵심 코드 | 0:25 |
| 5 | 하드웨어 | 0:20 |
| 6 | 진행 상황 | 0:20 |
| 7 | 상태 흐름 | 0:20 |
| 8 | 문제·해결 | 0:15 |
| 9 | 향후 계획 | 0:10 |
| 10 | 마무리 | 0:05 |
| 합계 |  | ~3:00 |

## 발표 팁 / Presentation Tips

슬라이드 6~7이 길면 슬라이드 8을 짧게 줄이세요.

PPT는 영어, 말은 한국어 대본으로 해도 됩니다. 영어 발표 시 Part 1 사용.

시연 영상이 있으면 슬라이드 5 또는 6에서 Lab 9 demo 언급 가능.

Q&A 예상: Why camera + ultrasonic? / Why no speaker? / Is LLM on Pi?